JINGWU ROBOT

Complete System Architecture & Component Guide

Deep Technical Explanation of Every Component — How It Works, Why It Exists, and How Everything Connects

# OVERVIEW: What Is the Jingwu Robot?

The Jingwu Robot is an autonomous service/delivery robot designed for indoor and outdoor environments. It uses a distributed electronics architecture split into four physical zones: the Front Face (sensors & actuators), the Display Part (human-machine interface), the Upper Compartment (communication, power, and main processing hub), and the Lower Compartment (mobility, power management, and high-level decision making). Each zone is carefully engineered so that components with similar roles are housed together, reducing cable runs and improving modularity.

Think of it like a human body: the Lower Compartment is the brain and heart (main controller + battery), the Upper Compartment is the nervous system (routing signals, power, and data), the Front Face is the sensory organs (eyes, ears, touch), and the Display Part is the face (communicating with humans).

# SECTION 1: FRONT FACE

The Front Face is the robot's primary perception and interaction surface — the part that faces the world. It contains all the sensors the robot uses to see, feel, and navigate its immediate environment, plus actuators for physical tasks.

## 1.1 TOF Sensor (Time of Flight) — x2 units

| Full Name | Time of Flight Distance Sensor |
| --- | --- |
| What It Does | Measures precise distance to objects by emitting infrared laser pulses and timing how long they take to return. |
| How It Works | Emits a burst of infrared (IR) light. The light hits an object and bounces back. The sensor measures the round-trip time and calculates: Distance = (Speed of Light × Time) / 2. |
| Why Two? | One points forward for obstacle detection; one may point downward for edge/drop detection (e.g., stair detection). |
| Real-World Example | iPhone Face ID uses TOF to map your face in 3D. Self-driving cars use LiDAR (industrial-grade TOF). Amazon warehouse robots use TOF to avoid collisions. |
| Accuracy | Typically ±1–3 cm, works in both light and dark environments, range 10 cm to 5 meters. |

Flow: TOF → Upper Controller Board (HUB) → Main Controller (OCT600) for collision avoidance decisions.

## 1.2 Camera

| What It Does | Captures visual images and video of the environment in front of the robot. |
| --- | --- |
| How It Works | A CMOS image sensor converts light into electrical signals. Each pixel records Red/Green/Blue intensity. Data is processed as frames (images per second) and sent via USB or MIPI interface to the controller. |
| Why Needed | Enables advanced functions: human face recognition for personalization, QR code / barcode scanning for deliveries, obstacle classification (telling a trash bin from a person), visual navigation landmarks, and video streaming for remote monitoring. |
| Real-World Example | Amazon Ring doorbell cameras. Self-checkout kiosks that scan items. Hospital robots like Moxi use cameras to identify where to deliver supplies. |
| Connection | Camera → Upper Controller Board (HUB) → OCT600 Main Controller for image processing. |

## 1.3 Ultrasonic Sensor

| What It Does | Detects obstacles using sound waves, complementing the TOF sensors. |
| --- | --- |
| How It Works | A piezoelectric element emits a 40 kHz ultrasonic pulse (inaudible to humans). The pulse bounces off objects. The sensor measures time-of-return. Distance = (Speed of Sound × Time) / 2. Speed of sound = 343 m/s at room temperature. |
| Why Use Both TOF and Ultrasonic? | TOF is precise but can fail on transparent or highly reflective surfaces (glass, mirrors). Ultrasonic works on almost any surface. Together they give redundant coverage for safety. |
| Real-World Example | Car parking sensors (the beeping when you reverse). Industrial robots use ultrasonic for close-range detection. Xiaomi robot vacuums use ultrasonic + infrared. |
| Range | Typically 2 cm to 4 meters, accuracy ±3 mm. |

## 1.4 LED

| What It Does | Visual status indicator and human communication tool. |
| --- | --- |
| How It Works | Light Emitting Diode driven by a GPIO (General Purpose Input/Output) pin from the controller. Can be driven with PWM (Pulse Width Modulation) to control brightness. RGB LEDs combine Red, Green, Blue channels to produce any color. |
| Why Needed | Robots must communicate state to nearby humans: Green = normal operation, Red = error/fault, Blue = charging, Flashing = processing or seeking attention. This is critical for human-robot safety trust. |
| Real-World Example | Tesla's indicator LEDs on Autopilot cameras. Boston Dynamics Spot uses LED eyes to signal its operational state. Hospital robots flash lights before moving. |
| Connection | Upper Controller Board (HUB) → GPIO pin → LED driver circuit → LED. |

## 1.5 Stepper Motor

| What It Does | Provides precise mechanical movement — likely controls a rotating tray, head, or delivery compartment door. |
| --- | --- |
| How It Works | A stepper motor divides a full 360-degree rotation into a fixed number of equal steps (typically 200 steps = 1.8 degrees per step). By energizing coils in sequence, it moves exactly one step at a time — no encoder needed for position feedback. This is called open-loop control. |
| Why Stepper vs Regular Motor? | Regular DC motors spin freely — you don't know their exact position. Stepper motors give exact, repeatable positioning. Perfect for opening/closing a delivery door to a specific angle, rotating a camera, or deploying a tray. |
| Real-World Example | 3D printers use stepper motors to move the print head with 0.1 mm precision. CNC machines. The rotating delivery tray on Starship delivery robots. |
| Driver | Controlled via Microstep Driver (see Section 3.4) for smoother motion. |

## 1.6 Limit Switch (x2)

| What It Does | Detects the physical end-of-travel position of a moving mechanism (e.g., door fully open or fully closed). |
| --- | --- |
| How It Works | A simple mechanical switch. When a moving part pushes the lever arm, the circuit opens or closes, sending a signal to the controller. This is called a 'hard stop' — a physical safety limit. |
| Why Two? | One detects the fully-open position, the other detects fully-closed. The controller knows exactly where the mechanism is at all times. |
| Real-World Example | Elevator doors have limit switches that detect when the door is fully open or closed. Garage door openers use them. Industrial robot arms have limit switches on every joint to prevent over-rotation. |
| Safety Role | Without limit switches, a motor would keep running past its endpoint, destroying gears or jamming mechanisms. Limit switches are a fundamental safety device. |

# SECTION 2: DISPLAY PART

The Display Part is the robot's human-machine interface (HMI) — how the robot 'speaks' to users. It is physically separate from the main computing area, connected via signal cables, allowing the screen to be positioned at human eye level.

## 2.1 Touch Controller

| What It Does | Detects and processes finger touches on the display screen, translating physical touch into digital coordinates. |
| --- | --- |
| How It Works | A capacitive touch controller (like the IC FT5426 or similar) overlays a transparent grid of electrodes on the screen. Your finger disrupts the electrical field. The controller chip calculates the X,Y coordinates and sends them via I2C or SPI bus to the main computer. |
| Why Separate Controller? | The touch processing chip offloads work from the main CPU. It handles noise filtering, multi-touch detection, and gesture recognition independently. |
| Real-World Example | Every smartphone touchscreen has a dedicated touch controller IC. ATMs, hospital patient call systems, airport check-in kiosks all use industrial touch controllers like this. |
| Interface | Touch Controller → I2C/SPI bus → Upper Controller Board (HUB) → OCT600. |

## 2.2 UT2660 V1 Display Module

| What It Is | An embedded display computer module — likely a combination of screen + embedded processor running the UI software. |
| --- | --- |
| How It Works | The UT2660 V1 appears to be an all-in-one display unit with its own processor, running a Linux or Android-based UI. It renders graphics (robot face expressions, menu options, delivery status), plays audio, and accepts touch input. Communicates with the main controller via USB, UART, or Ethernet. |
| What It Shows | Robot status messages (Ready, Delivering, Charging), interactive menus for users, animated face expressions for friendly human interaction, delivery notifications and confirmations, video or image content. |
| Real-World Example | The screens on Pepper robot (SoftBank) that show its face. McDonald's self-order kiosks. Hospital bedside terminals. The display on Amazon Astro home robot. |
| Why Important | Studies show robots with expressive displays are trusted more by humans. A clear UI reduces errors in human-robot interaction — critical for delivery tasks. |

# SECTION 3: UPPER COMPARTMENT

The Upper Compartment is the robot's communication and power distribution hub. It takes raw 24V-48V battery power, conditions it, routes it to all subsystems, and manages all network communication between components and the outside world.n

## 3.1 100W DC-DC Converter

| What It Does | Steps down the main battery voltage (typically 24V or 48V) to lower voltages needed by electronics (12V, 5V, 3.3V). |
| --- | --- |
| How It Works | A switch-mode power supply (SMPS) uses a high-frequency switching transistor (50kHz-500kHz), an inductor, and a capacitor to transfer energy efficiently. Unlike linear regulators that waste energy as heat, DC-DC converters achieve 85-95% efficiency. |
| Why 100W? | The upper compartment electronics (router, controller board, display, sensors) together draw significant power. 100W provides headroom: 4G router ~10W + Controller board ~15W + Display ~20W + sensors ~5W = ~50W total, with 50W safety margin. |
| Real-World Example | The power brick for your laptop is a DC-DC converter. Electric vehicles use DC-DC converters to power 12V accessories from the 400V battery pack. Server racks use them for every board. |
| Connection | Battery Pack → 100W DC-DC Converter → Powers the 4G LTE Router and DC-DC Isolated Power Supply. |

## 3.2 DC-DC Isolated Power Supply

| What It Does | Provides galvanically isolated (electrically separated) power to sensitive electronics, protecting them from voltage spikes and noise. |
| --- | --- |
| How It Works | Uses a transformer to transfer energy between circuits with NO direct electrical connection between input and output. This means a voltage spike on the battery side cannot damage the microcontrollers or sensors. Isolation is measured in kV (typically 1kV-4kV isolation). |
| Why Critical? | Motor drive systems generate electrical noise (EMI — electromagnetic interference) on the power lines. Sensors and microcontrollers are extremely sensitive to this noise. Isolated power supply acts as a clean, noise-free power source for all precision electronics. |
| Real-World Example | Medical equipment MUST use isolated power supplies to protect patients. Industrial robots use isolated supplies for their control electronics. Every EV charger has an isolated DC-DC converter between grid and car battery. |
| Connection | 100W DC-DC Converter → DC-DC Isolated Power Supply → Upper Controller Board (HUB) + sensors. |

## 3.3 4G LTE Router

| What It Does | Provides cellular internet connectivity for the robot, enabling remote monitoring, cloud navigation updates, fleet management, and remote control. |
| --- | --- |
| How It Works | Contains a cellular modem (SIM card slot) that connects to the 4G LTE mobile network. Acts as a WiFi access point and wired Ethernet router for all internal components. Provides a private IP subnet inside the robot, and routes traffic to the internet through the cellular connection. |
| Why 4G vs WiFi Only? | WiFi requires fixed infrastructure (routers in the building). 4G works anywhere there is cellular coverage — outdoors, across buildings, in areas with no WiFi. Critical for outdoor delivery robots. |
| Capabilities | Remote monitoring dashboard, OTA (Over-the-Air) software updates, real-time fleet tracking via GPS data relay, emergency remote stop command, log upload for diagnostics. |
| Real-World Example | Amazon Scout delivery robot uses cellular. Starship delivery robots stream GPS data via cellular. Industrial AGV fleets (Amazon warehouse robots) use a mix of WiFi and cellular. |

## 3.4 Upper Controller Board (HUB)

| What It Is | The central nervous system of the Upper Compartment — a multi-purpose controller board that aggregates sensor data, distributes commands, and interfaces with all upper-body components. |
| --- | --- |
| How It Works | Contains a microcontroller or SBC (Single Board Computer) with multiple communication interfaces: CAN Bus (for real-time motor/sensor communication), USB (for camera, display), UART/Serial (for sensor modules), I2C/SPI (for touch controller, small sensors), Ethernet (for data to main controller OCT600). Acts as a protocol converter and data multiplexer. |
| Why a Dedicated HUB? | If every sensor connected directly to the main controller, it would be overwhelmed with low-level I/O. The HUB pre-processes data, reducing the main controller's load. This is called a distributed architecture. |
| Real-World Example | In a Boeing aircraft, there are dedicated data concentration units that aggregate sensor data before sending to the flight computer. Tesla Autopilot has dedicated sensor fusion computers (HW3/HW4 chips). Industrial robots (KUKA, ABB) use dedicated I/O modules. |
| Connects To | Receives from: TOF, Camera, Ultrasonic, LED, Stepper (via Microstep Driver), Touch Controller, Wireless Transceiver. Sends to: OCT600 Main Controller (via CAN/Ethernet), Microstep Driver. |

## 3.5 Wireless Transceiver

| What It Does | Provides short-range wireless communication — likely for local mesh networking, remote controller input, or communication with charging stations. |
| --- | --- |
| How It Works | Typically uses 2.4GHz or 915MHz radio frequency. Could be ZigBee, Sub-GHz LoRa, or proprietary RF protocol. The transceiver converts digital data packets into radio waves and vice versa. Has its own microcontroller for protocol handling. |
| Use Cases | Receiving commands from a handheld remote control during setup/testing, communicating with a docking/charging station (tell it when to activate charging contacts), short-range mesh networking with other robots in a fleet, emergency stop signal from a dedicated safety transmitter. |
| Real-World Example | Warehouse robots communicate via 900MHz radio for reliable penetration through metal shelving. Hospital robots use wireless transceivers to communicate with elevator control systems to call elevators automatically. |

## 3.6 Microstep Driver

| What It Does | Drives the Stepper Motor in the Front Face with precise micro-stepping control for smooth, quiet movement. |
| --- | --- |
| How It Works | A standard stepper motor has 200 full steps per revolution (1.8 degrees each). A microstep driver divides each full step into 8, 16, or 32 smaller micro-steps. Instead of jumping 1.8 degrees at once, it moves 0.1 degrees at a time, producing MUCH smoother motion and eliminating vibration. Controls motor current via PWM to energize coils in precise ratios. |
| Why Important? | Full-step driving causes vibration and noise that can disturb people (robot is a service robot near humans). Micro-stepping makes movement near-silent and smooth, which is critical for professional impression. |
| Real-World Example | High-end 3D printers (Prusa, Bambu Lab) use micro-stepping drivers (TMC2209 chips) to achieve silent printing. CNC routers use micro-stepping for precise cuts. Camera lens autofocus systems use micro-stepper motors. |
| Connection | Upper Controller Board (HUB) → STEP/DIR signals → Microstep Driver → Stepper Motor current → Motor. |

# SECTION 4: LOWER COMPARTMENT

The Lower Compartment is the robot's brain and power core. It houses the main computational controller, the battery, the chassis (wheel) control, charging management, and safety systems. This is the most critical section — if anything here fails, the robot stops.

## 4.1 OCT600 Main Controller

| What It Is | The robot's primary CPU/computing platform — the brain that runs all high-level software including navigation algorithms, task planning, and system coordination. |
| --- | --- |
| How It Works | The OCT600 is likely an embedded computing platform (ARM-based SBC or custom industrial computer) running a real-time OS (Linux with RT kernel or ROS — Robot Operating System). It receives processed sensor data from the Upper Controller HUB, runs SLAM (Simultaneous Localization and Mapping) for navigation, plans paths using A* or Dijkstra algorithms, sends motion commands to the Chassis Controller Board, and manages task queues (delivery jobs). |
| Key Software Functions | SLAM navigation (building a map while navigating), obstacle avoidance path planning, fleet communication with cloud server, task management (where to go, what to pick up), health monitoring of all subsystems. |
| Real-World Example | The NVIDIA Jetson computer in Boston Dynamics Spot. The Intel NUC in ROS-based research robots. The main compute unit in autonomous forklifts. Tesla's Full Self-Driving computer running neural networks. |
| Connection | Connects to: Upper Body Controller (for LIDAR data), Chassis Controller Board (for wheel commands), Battery Pack (for power), Control Panel (for manual input), LIDAR (direct connection for mapping). |

## 4.2 Upper Body Controller

| What It Does | Acts as a sub-controller managing the upper body sensors and modules, with a direct connection to the LIDAR sensor for rapid map updating. |
| --- | --- |
| How It Works | Runs a lightweight real-time OS. Continuously reads LIDAR scan data (which arrives at 10-20Hz), pre-processes it (filters noise, converts polar to Cartesian coordinates), and passes clean data to the OCT600. Also coordinates upper body LED status indicators and manages the overall upper body power state. |
| Why Separate From OCT600? | LIDAR generates massive amounts of data (360 range measurements every 100ms). Processing this in real-time requires a dedicated processor to avoid slowing down the main navigation algorithms on OCT600. |
| Real-World Example | In NASA Mars rovers, separate controllers handle different subsystems (arm, wheels, instruments) while a central computer handles navigation. This prevents one subsystem's failure from crashing the entire robot. |

## 4.3 LIDAR (Light Detection and Ranging)

| What It Does | Scans the environment in a full 360-degree horizontal sweep to build a precise 2D map and detect obstacles at long range. |
| --- | --- |
| How It Works | A rotating laser emitter spins 360 degrees (typically 5-15 times per second). At each angle, it fires a laser pulse and measures the time-of-flight to calculate distance. This creates a point cloud — thousands of distance measurements forming a precise 2D slice of the environment. Common LIDAR for service robots: SLAMTEC RPLIDAR A3, Hokuyo UST-10LX. |
| Why LIDAR Is the Core Navigation Sensor | LIDAR creates a precise, meter-accurate map in real time. It works in darkness, unlike cameras. Range of 8-25 meters gives the robot time to plan paths smoothly. Used for SLAM — the robot simultaneously builds a map of its environment and knows where it is within that map. |
| Real-World Example | Waymo self-driving cars use 64-channel 3D LIDAR. iRobot Roomba j-series uses a simplified LIDAR. All major AMR (Autonomous Mobile Robot) warehouse fleets (Locus Robotics, 6 River Systems) use 2D LIDAR for navigation. |
| Connection | LIDAR → Upper Body Controller → OCT600 for SLAM processing. |

## 4.4 Chassis Controller Board

| What It Does | The real-time motor controller for the drive wheels — translates high-level navigation commands (Go forward 1 meter, Turn 90 degrees left) into precise wheel motor commands. |
| --- | --- |
| How It Works | Contains a real-time microcontroller (STM32 or similar) running a PID (Proportional-Integral-Derivative) control loop. Receives velocity commands from OCT600. Reads wheel encoder feedback (actual speed). Calculates the error (desired vs actual speed). Adjusts PWM signal to motor drivers to correct the error. This closed-loop control makes the robot move with consistent speed even on slopes or different floor surfaces. |
| PID Explained Simply | If the robot is going slower than commanded (uphill), PID increases motor power. If going too fast (downhill), it reduces power. This happens hundreds of times per second. |
| Real-World Example | Electric vehicle motor controllers use PID loops. CNC machine axis controllers. Every servo motor in an industrial robot arm uses a PID controller. |
| Connects To | OCT600 (receives commands), Wheel Encoder Interface (receives feedback), motor drivers (sends PWM), Battery Pack (power). |

## 4.5 Battery Pack

| What It Does | Provides all electrical power for the entire robot system. |
| --- | --- |
| How It Works | Lithium Iron Phosphate (LiFePO4) or Lithium-Ion battery pack. Typical service robot battery: 24V-48V, 20-50Ah capacity (480Wh-2400Wh). Contains a BMS (Battery Management System) that monitors cell voltages, temperature, state of charge (SOC), and protects against overcharge, over-discharge, and short circuit. Communicates battery status (voltage, current, SOC, temperature) to the OCT600 via CAN Bus or UART. |
| Battery Life Calculation | A 1000Wh battery powering a 100W system = 10 hours runtime. Service robots typically target 8-12 hours per charge. |
| Why LiFePO4? | Safer (no thermal runaway risk unlike standard Li-Ion), longer cycle life (2000+ charge cycles vs 500 for Li-Ion), better performance at temperature extremes. |
| Real-World Example | Electric golf carts, AGV warehouse robots (Toyota, Crown), Starship delivery robots all use LiFePO4 packs. Tesla Powerwall home battery uses LiFePO4. |
| Connection | Distributes power to: OCT600, Chassis Controller Board, Charging Control Board, Upper Compartment (via 100W DC-DC converter). |

## 4.6 Charging Control Board

| What It Does | Manages the battery charging process — controls how current flows into the battery pack during charging, protecting it from overcharging. |
| --- | --- |
| How It Works | Implements CC-CV charging (Constant Current - Constant Voltage): Phase 1 (CC): Charges at a fixed current rate (e.g., 10A) until battery reaches 80% SOC. Phase 2 (CV): Holds voltage constant while current tapers off as battery fills to 100%. Monitors temperature and reduces charging rate if too hot. Communicates charging status to OCT600. |
| Why Dedicated Board? | Proper battery charging is complex and safety-critical. A dedicated board with precise analog circuits handles this more reliably than a general-purpose controller. |
| Real-World Example | Your phone's fast-charging system (Qualcomm Quick Charge) uses a CC-CV algorithm. Tesla Supercharger uses an advanced version of this. All AGV auto-charging docking stations have charging controllers. |
| Connects To | Battery Pack (charges it), Charging Contact (receives power), Control Panel (status display), OCT600 (reports status). |

## 4.7 Charging Contact

| What It Does | Physical electrical contacts that connect to a charging dock for automatic charging without a plug. |
| --- | --- |
| How It Works | Spring-loaded metal contact pads on the robot's underside or rear. When the robot docks at the charging station, contacts align with the station's charging pins. A small current is first used to verify alignment and identity before full charging current flows. This is called 'contact docking charging.' |
| Why No Plug? | Plugging a cable requires precise manipulation (hard for a robot). Auto-docking contacts work reliably within ±5mm alignment tolerance. The robot can autonomously return to charge when battery is low. |
| Real-World Example | iRobot Roomba's charging dock uses contact pads. Amazon Scout uses contact charging. Autonomous patrol robots in airports auto-dock for charging between patrols. |

## 4.8 Wheel Encoder Interface

| What It Does | Reads the wheel encoder sensors and provides precise wheel rotation data (speed and direction) to the Chassis Controller Board for closed-loop control. |
| --- | --- |
| How It Works | Incremental encoders on each wheel generate pulses as the wheel turns — typically 500-4096 pulses per revolution. The interface board counts these pulses and calculates RPM and direction. Provides odometry data (distance traveled) to the OCT600 for dead-reckoning navigation (estimating position from wheel movements). |
| Why Separate Interface Board? | Encoder signals are high-frequency (up to 100kHz at speed). A dedicated interface board handles interrupt-driven counting accurately, without missing pulses, even when the main controller is busy. |
| Odometry | By knowing wheel circumference and counting rotations, the robot calculates exactly how far it has moved. Combined with LIDAR SLAM, this gives centimeter-accurate positioning. |
| Real-World Example | All wheeled industrial robots use encoder-based odometry. Forklift AGVs in warehouses navigate with ±5mm accuracy using encoders + LIDAR. |

## 4.9 Control Panel

| What It Does | A physical human interface panel (buttons, switches, indicator lights) for direct operator control — used for startup, emergency stop, mode selection, and maintenance. |
| --- | --- |
| How It Works | Physical buttons connect to GPIO inputs on OCT600 or Chassis Controller. Includes: Emergency Stop button (hardwired, cuts motor power instantly), Power On/Off switch, Mode selector (Auto/Manual/Charging), Status LEDs, possibly a small LCD showing system status like battery level. |
| Why Physical Panel? | Software interfaces can fail. A physical E-Stop that directly cuts power is a fundamental robot safety requirement (ISO 10218 standard for robot safety). Physical buttons work when software is frozen or rebooting. |
| Real-World Example | All industrial robots (KUKA, ABB, Fanuc) have physical teach pendants and E-Stop buttons. CNC machines have physical emergency stops. Hospital robots have manual override switches for staff. |

# SECTION 5: COMPLETE SYSTEM FLOW — Step by Step Operation

Here is how all components work together for a typical delivery task:

## Step 1: System Startup

- Operator presses Power button on Control Panel.

- Battery Pack begins supplying power to OCT600 and all subsystems.

- 100W DC-DC Converter + Isolated Power Supply powers upper compartment electronics.

- OCT600 boots Linux/ROS operating system (~30-60 seconds).

- Upper Body Controller initializes LIDAR — begins 360-degree scanning.

- Upper Controller HUB initializes: TOF, Camera, Ultrasonic sensors come online.

- UT2660 Display shows boot animation, then 'Ready' screen.

- 4G LTE Router connects to cellular network, registers robot with fleet management server.

## Step 2: Receiving a Delivery Task

- Cloud server sends delivery task via 4G LTE Router to OCT600.

- OR: User enters destination on UT2660 Touch Display.

- OCT600 loads the facility map (pre-built via SLAM), plans optimal path using A* algorithm.

- OCT600 sends 'move to location X' command to Chassis Controller Board.

## Step 3: Navigation in Motion

- Chassis Controller Board sends PWM signals to wheel drive motors.

- Wheel Encoder Interface reads encoder pulses → sends real speed back to Chassis Controller.

- PID loop corrects motor speed 1000 times/second for stable movement.

- LIDAR continuously scans 360 degrees → Upper Body Controller processes point cloud.

- OCT600 runs SLAM: compares live LIDAR scan to stored map → calculates robot position.

- TOF sensors + Ultrasonic sensor monitor close-range obstacles ahead.

- Camera checks for dynamic obstacles (people, other robots) using computer vision.

- If obstacle detected: OCT600 recalculates path → sends new commands to Chassis Controller.

## Step 4: Human Interaction at Destination

- Robot arrives at destination. Stepper Motor (via Microstep Driver) opens delivery compartment.

- Limit Switch detects compartment fully open → signals Upper Controller HUB → OCT600 confirms.

- UT2660 Display shows recipient notification screen.

- Touch Controller accepts recipient's confirmation tap.

- LED flashes to attract attention.

- After confirmation, Stepper Motor closes compartment. Limit Switch confirms closure.

## Step 5: Return and Auto-Charging

- OCT600 monitors battery SOC from Battery Pack BMS via CAN Bus.

- When battery reaches 20% threshold, OCT600 plans route to charging dock.

- LIDAR + camera guide robot precisely onto charging dock alignment (within ±5mm).

- Charging Contact touches dock pins. Charging Control Board detects valid connection.

- CC-CV charging begins. Charging Control Board monitors voltage/current/temperature.

- OCT600 reports charging status to cloud server via 4G Router.

- When 100% charged, Charging Control Board signals OCT600. Robot reports 'Ready' status.

# SECTION 6: Communication Buses and Protocols

The robot uses multiple communication standards, each chosen for its specific strengths:

| Protocol | Speed | Used For |
| --- | --- | --- |
| CAN Bus | 1 Mbps | OCT600 ↔ Chassis Controller ↔ Battery BMS ↔ Charging Board. Real-time, fault-tolerant, used in all automotive/industrial robots. |
| Ethernet | 100Mbps+ | OCT600 ↔ Upper Controller HUB ↔ 4G Router. High-speed data: camera streams, LIDAR data, software updates. |
| UART/Serial | Up to 1Mbps | Controller ↔ Wireless Transceiver, Controller ↔ GPS, Controller ↔ LIDAR (some models). |
| I2C | 400kHz | Short-range chip-to-chip: Touch Controller ↔ HUB, IMU ↔ HUB, sensor ICs. |
| SPI | Up to 50MHz | Fast sensor data: some LIDAR models, display framebuffer. |
| GPIO | N/A | Simple on/off signals: Limit Switches, LED control, E-Stop detection. |
| PWM | N/A | Motor speed control: Microstep Driver signal, DC motor driver signals, LED dimming. |

# SECTION 7: Safety Architecture

The Jingwu robot implements multiple layers of safety — this is called Defense in Depth:

## Layer 1 — Physical Safety (Hardware)

- Limit Switches: Prevent mechanical over-travel — hardwired, no software needed.

- E-Stop on Control Panel: Directly cuts motor power — bypasses all software.

- Charging Contact polarity protection: Prevents reverse-connection damage.

- Battery BMS: Prevents overcharge, over-discharge, short circuit at hardware level.

## Layer 2 — Real-Time Safety (Firmware)

- PID controller watchdog: Chassis Controller detects if OCT600 stops sending commands and auto-stops wheels.

- Encoder monitoring: If wheel stalls (obstacle jammed against wheel), detects motor stall and stops.

- Isolated power supply: Protects control electronics from motor noise spikes.

## Layer 3 — Software Safety (OS/Application)

- TOF + Ultrasonic close-range emergency stop: If obstacle detected within 20cm, immediate halt.

- LIDAR safety zones: Configurable zones where robot slows or stops.

- Camera person detection: Slows robot in presence of humans.

- Health monitoring: OCT600 monitors all subsystem heartbeats; if any component stops responding, robot parks safely and alerts fleet server.

# QUICK REFERENCE: All Components Summary

| Component | Zone | Core Function |
| --- | --- | --- |
| TOF Sensor x2 | Front Face | Precise close-range distance measurement using infrared laser time-of-flight |
| Camera | Front Face | Visual perception: people detection, QR scanning, visual navigation |
| Ultrasonic Sensor | Front Face | Redundant obstacle detection via sound waves, works on any surface |
| LED | Front Face | Visual status indicator and human communication |
| Stepper Motor | Front Face | Precise mechanical movement: door/tray/compartment actuation |
| Limit Switch x2 | Front Face | Physical end-of-travel detection for mechanical safety |
| Touch Controller | Display | Translates finger touch to digital coordinates for UI interaction |
| UT2660 V1 Display | Display | Embedded display computer: shows UI, robot face, delivery info |
| 100W DC-DC Converter | Upper | Efficient battery voltage step-down to operating voltages |
| DC-DC Isolated PSU | Upper | Clean, noise-isolated power for sensitive electronics |
| 4G LTE Router | Upper | Cellular internet: cloud connectivity, remote monitoring, OTA updates |
| Upper Controller HUB | Upper | Sensor aggregation and protocol conversion hub |
| Wireless Transceiver | Upper | Short-range RF: dock communication, remote control, mesh networking |
| Microstep Driver | Upper | Smooth, precise stepper motor drive with micro-step resolution |
| OCT600 Main Controller | Lower | Central brain: SLAM navigation, task planning, system coordination |
| Upper Body Controller | Lower | LIDAR data pre-processing, upper body subsystem management |
| LIDAR | Lower | 360-degree laser mapping for SLAM navigation |
| Chassis Controller Board | Lower | Real-time wheel PID motor control for precise movement |
| Battery Pack | Lower | Main energy storage with BMS for safety and monitoring |
| Charging Control Board | Lower | CC-CV battery charging management and protection |
| Charging Contact | Lower | Auto-docking electrical contacts for autonomous recharging |
| Wheel Encoder Interface | Lower | Wheel rotation counting for odometry and PID feedback |
| Control Panel | Lower | Physical operator interface: E-Stop, power, mode, status |

End of Jingwu Robot Technical Documentation